IR Sensor Connections

Left IR sensor:

- VCC → Arduino 5V

- GND → Arduino GND

- OUT → Arduino A0 (lefts A0)

Right IR sensor:

- VCC → Arduino 5V

- GND → Arduino GND

- OUT → Arduino A1 (rights A1)

Motor Shield Connections

Motor 1 (left wheel motor):

- Connect to M1 terminal of Motor Shield

Motor 2 (right wheel motor):

- Connect to M2 terminal of Motor Shield

Battery:

- Motor battery (+) → Motor Shield external power terminal (+)

- Motor battery (−) → Motor Shield GND

Motor shield sits directly on top of Arduino.

Working Logic

Sensor values:

- Black line ≈ 1017

- White surface ≈ 38

Robot behavior:

| Left Sensor | Right Sensor | Action |
| --- | --- | --- |
| Black | Black | Stop |
| Black | White | Turn Right |
| White | Black | Turn Left |
| White | White | Move Forward |